# Note: $ is a flag that there is something to check, fill in, or address. I use $ since it is easy to search for as my scientific papers usually don’t include prices.

Personally I don’t like footnotes much, they always look like an answer to reviewers, I think most of the text could be included into the body.

# Relations between radar reflectivity, liquid water content,

# and rainfall rate during the MAP-SOP

Martin Hagen* and Sandra E.Yuter**

*  Deutsches Zentrum für Luft- und Raumfahrt (DLR), Institut für Physik der Atmosphäre, Weßling, Germany

** Department of Atmospheric Sciences, University of Washington, Seattle, Washington, USA

Summary

Rain drop size distribution data obtained from two Joss-Waldvogel disdrometers located at Locarno-Monti, Switzerland during MAP are analyzed to obtain appropriate Z-W and Z-R relationships for use in MAP applications.  The disdrometer data are accumulated into 10 min samples to reduce sampling error associated with the ~1 m3 sample volume of the instrument.  Based on previous studies, relations of the form W=qZ(4/7)  and Z=aR1.5 are assumed and the coefficients q and a are estimated from the data.  The combined data set of 10 min samples from the two disdrometers and the 10 min average data yielded nearly identical mean values of the coefficients.  The recommended relationships are W=3.4Z (4/7) $ I’m confused about what I saw in literature, should this be 3.4e-3 ?$ and Z=216R1.5 .  The uncertainties in these mean relationships as expressed in terms of  1 standard deviation are approximately equivalent to a  4.4 dBZ error for the Z-W relationship,  and to a  2.4 dBZ error for the Z-R relationship.

# 1. Introduction

Horizontal maps of near-surface rainfall are important in understanding the water cycle of a region and in applications such as flood forecasting, fresh water management, and detection of climate change. Scanning weather radars yield maps of radar reflectivity (Z) which be used to estimate surface rainfall (R).  The relationship between measured radar reflectivity and surface rainfall (R) is complex and the estimation procedure is subject to several independent sources of error (Austin 1987, Joss and Lee 1995).   Biases can result from errors in radar calibration, non-meteorological echo such a ground clutter and anomalous propagation, the vertical variation of reflectivity in the storm between the measurement several km above the surface and the surface, the presence of non-rain hydrometeors such as graupel, hail, and melting snow, attenuation, and variations in the rain drop size distribution (RDSD).  Of these potential sources of bias, all but the last two can be removed or minimized by established methods. For the purposes of this paper, we will assume that such procedures are utilized, and explore the relatively smaller magnitude biases due to the variation in RDSD (Joss and Lee 1995). Nevertheless, the variation of the RDSD are the reason for the wide variety of Z-R relations wich are known from literature.

An estimate of three-dimensional liquid water content (W) $ should we call it rain water content, since we don’t see the cloud drops with the disdromter or a X/C/S-band radar ?? $of the volume can be obtained when radars scan several elevation angles to obtain a three-dimensional volume of radar reflectivity.  Volumetric liquid water content can be useful in the initiation and validation of numerical models. The Z-W estimation procedure has all the sources of error associated with the estimation of surface rainfall except for the vertical variation in Z since a transformation to near surface values is not required.

During the Mesoscale Alpine Programme Special Observing Period (MAP-SOP) (Bougeault et al. 2001), the rain drop size distribution within orographic precipitation was measured using two disdrometers deployed at the MeteoSwiss Osservatorio Ticinese in Locarno-Monti, Switzerland.  These data are analyzed to estimate appropriate Z-R and Z-W relations for the MAP-SOP.

# 2. Data

A disdrometer measures drop size distribution by counting the number of drops within each of several size categories over a time interval. We used two Joss-Waldvogel disdrometers (Joss and Waldvogel, 1967, Waldvogel 1974) one operated by the University of Washington (UW) and the other operated by the Deutsches Zentrum für Luft- und Raumfahrt (DLR) Institut für Physik der Atmosphäre.  The UW instrument was the standard RD-69/ADA-90 instrument. The DLR instrument was the RD-69 instrument and was used together with a self-constructed RDSD analyser. The Joss-Waldvogel disdrometer is an electro-mechanical instrument.  The momentum of a raindrop falling at its terminal velocity on a styrofoam cone with area 50 cm² is converted to an electrical impulse. The amplitude of this impulse is proportional to the diameter of the raindrop. The instruments utilizes 20 size categories to measure drops from ~0.3 to ~5 mm diameter for the UW disdrometer and ~0.5 to ~5 mm for the DLR disdrometer, respectively. Drops smaller than ~0.3 mm do not produce an impulse sufficiently above the noise level. Larger raindrops are all grouped into the last of the 20 classes. The size categories for the DLR disdrometer were calibrated by measuring the transfer function of the signal processing electronics (Sheppard 1990). $ with my recalibration this is now obsolete, I got the good agreement without further adjustment. I should have done this recalibration earlier $ The UW disdrometer used the factory calibration and standard diameter categories supplied by the instrument manufacturer, Distromet Inc.

At larger rain rates, the detection efficiency for small drops is further reduced due to the generation of environmental noise by the rain itself and an accompanying increase in the magnitude of the noise level below which drops cannot be detected (Joss and Gori 1976).  A short “dead-time” is built into the instrument so that splashes associated with the impact of a large drop on the sensor are not counted as small drops within the RDSD. However, during this dead-time, neither splash products nor actual small drops $ any drop ?? but small are more likely $in the RDSD are measured. In order to account for the small drops in the RDSD that were missed, a dead-time correction which is a function of the number and size of drops measured by the instrument is applied (see Appendix). $see my comments ther $

As a data quality check, both disdrometers were compared to a nearby MeteoSwiss rain gauge. $ Moved to above $ Table 1 shows the daily rainfall accumulations computed from the MeteoSwiss rain gauge and the two disdrometers.  A total of 862 mm was recorded during this three months. Overall, the instruments agreed well. Rain accumulations for both disdrometers were within 10% of the rain gauge for all days with rainfall over 10 mm. For the four days with less than < 1 mm rainfall measured by the disdrometers, the difference among the instruments was less than 0.2 mm. The largest discrepancies occurred on 22 October 1999 and 3 November and neither disdrometer registered any rainfall on 18 November while the gauge measured 0.7 mm. Possible causes of these discrepancies are under investigation. ($ for the 22 Oct I found two explanations: One is just the time around midnight, the rain gauge uses time intervals h-20 to h+40, I used h-60 to h, the other is: the distribution is very steep with a large number of small drops, the UW starts at 0.3 mm, the DLR starts at 0.5 mm. I didn’t find similar for the 3 Nov. On 18 Nov only drops below 0.6-0.7 mm were seen – see raw data – but even for the 60-min data this could not pass the 0.2 mm/h threshold $) The incomplete records on the UW disdrometer were the result of a computer rather than an instrument problem.

# 3. Methodology

The analysis of RDSD data collected by disdrometer must take into account the degree of representivity of the measurements in terms of their location and scale, and address statistical sampling error.

## a. Representivity of location

Locarno-Monti was within the Laggio-Maggiore Target Area (LMTA) of focussed observations designed to address the precipitation-related objectives of MAP (Bougeault et al. 2001) and is near a climatological local maxima of heavy precipitation in the southern Alps (Frei and Schär 1998).  Locarno-Monti received 30 days of rainfall during the period 20 September–18 November 1999 within a variety of synoptic conditions (Bougeault et al. 2001) and was near the center of the maximum rainfall accumulation during the MAP IOP2b event on 19-20 September 1999 (Rotunno and Ferretti 2002). The details of the rainfall distribution varied within the LMTA so one location cannot be exactly representative of other locations within the LMTA or the LMTA area mean.

## b. Representivity of spatial scale

The spatial scale of the recorded 1 min disdrometer measurements is order 1 m3. The spatial scale of the radar measurements to which they are intended to be applied is ~1 km3.  The order 109 difference in spatial scales is staggeringly large. It would take almost 31.7 ($ I get 1902 years: 1e9 / 60 min / 24 hr / 365 days$) years for a single disdrometer to measure a volume of atmosphere equivalent to a typical individual radar resolution volume. To date, all in situ measurements of the RDSD either via aircraft particle probes or surface-based disdrometers have had a sampling volume of 10 m3 or less. Without instantaneous in situ observations at larger scales, it has been difficult to assess how well the variability of the RDSD in time represents its variability in space or how well averaging in time represents averaging in space.

Joss and Gori (1978) examined the characteristics of the RDSD over increasing time periods within two storms at Locarno-Monti and found that after several hundred minutes the characteristics of the RDSD tended to converge toward an exponential distribution. A single instrument sample over 100’s of minutes in length is obtained within different portions of the storm and possibly is a result of different precipitation processes. Joss and Gori (1978) recognized this limitation. They concluded that " true exponential distributions are obtained when adding many 1 min samples of different rain intensity". Joss and Gori also found that the rate of change of the RDSD shape was not constant but varied approximately with the natural logarithm of the accumulation time. For example, the relative difference in average shape of the RDSD between samples for 1 min and 10 min accumulations was larger than between samples for 11 min and 20 min accumulations. In examining the issue of uniformity of precipitation processes, Kostinski and Jameson (1997) analyzed disdrometer time series data and found ~10 min duration rain "patches" with a similar number of drops of a given size per minute.  They described the RDSD at larger scales that would incorporate multiple rain "patches" as mixtures of Poisson distributions.

## c. Sampling error

Smith et al. (1993) modeled sampling errors in a ($ ?) normalized exponential RDSD as a means to assess the relative contributions of sampling uncertainties versus natural inhomogenities to the apparent variability of in situ RDSD measurements. They found a consistent low bias in estimates of  R and Z that decreased as the total number of drops in the sample increased. The low bias is a result of the mismatch between the typical measurement sample volume of 1 m3 and the average concentration of larger drops in the sample which is often less then 1 per 1 m3. For example, for an average concentration of 4 mm diameter drops of 1 drop per 100 m3, on average 99 of 100 1 min samples will not register a drop 4 mm in size . Without the large drop, the 99 samples will have a low bias in R and a slightly larger low bias in Z because of the ~D4 compared to D6 weighting. The one sample with the 4 mm drop will have high biases in R and Z, but when averaged with the other 99 samples, the mean bias will be still be low. This type of sampling bias associated with an exponential-type distribution where significant contributions to R and Z can come from low concentrations of large drops is in addition to the Poisson uncertainty which is based on the number of drops measured.

## d. Processing procedure

To process the disdrometer data to reduce uncertainties we have to compromise between two conflicting constraints. To reduce sampling error we should increase the number of drops by increasing the sampling accumulation time. To reduce errors associated with mixing samples representing distinct precipitation processes we should keep the sampling time small. As a compromise between these two constraints, we have chosen a 10 min accumulated RDSD as the basis of our analysis and a 60 min accumulated RDSD for comparison. A 10 min accumulation period allows us to reduce but not eliminate sampling errors. A 60 min accumulation period permits us to reduce sampling error further but at the expense of mixing rain patches. Since we are comparing data obtained from two instruments, we have the additional constraint that we would like to compare the same time periods, e.g. 01:00:00-01:09:59. This latter constraint means that sometimes we will include minutes within the 10 minute period where an individual instrument did not measure any drops. A time period is considered rainy if at least 80% of the 1 min measurements within the period had drops. In processing the data, we have removed 1 min measurement with less than 20 drops which usually correspond to non-precipitation triggers such as wind hits and insects. We have also applied a minimum rain rate threshold 0.2 mm/hr to remove accumulated samples prone to large sampling errors. ($I am no longer sure this threshold was the right thing to do, what we intend is a proxy for Ntot. I am thinking esp. in context of the 60 min samples which would have plenty of raw counts at rain rates < 0.2 mm/hr. Perhaps we should just threshold on Ntot>=100 for the accumulated sample instead, it wont change things much but all the numbers would have to be recomputed. One argument for not redoing the analysis would be that we do not concentrate so much on the weak rainfall (< 0.2 mm/h) This would increase the scatter at the low rain rate end of a Z-R relation plot. I checked my programm: the counts > 20 is done for each 1-minute sample, the 0.2 mm/h threshold is for the total time interval (10 or 60 minutes)$)

Radar reflectivity assuming Rayleigh scattering, liquid water content, and rain rate were calculated from the dead-time corrected RDSD (N(D) in units of # per m3 per mm) as follows.

Z=i=120N(Di)Di6ΔDi						[1]

W=π6i=120N(Di)Di3ΔDi						[2]

R=3.6π6000i=120N(Di)Di3V(Di,T,P)ΔDi				[3]

For each of the 20 size categories, Di is the mean diameter of the size category in mm, and Di is the width of the size category in mm. The units of Z are mm6 m-3, W are mm3m-3 and for R are mm hr-1 . The particle fall speed (V ) is a function of diameter, temperature, and pressure (Berry and Pranger 1974) and is in units of m s-1.

For our analysis below we used three versions of the disdrometer data, the union of the 10 min accumulated DLR and UW data (combo10), the union of the 60 min accumulated DLR and UW data (combo60), and the time-synchronized average of the 10 min DLR and UW data (aver10).  Table 2 shows statistics for the the aver10 data set and for the DLR and UW 10 min data sets separately. The effect of the dead-time of the instrument is evident in the smaller number of measured drops at higher rain rates. At least half of the rain accumulation was obtained within rain rates < 10 mm hr-1. The 60 min data has similar total accumulations but lower average rain rates compared to the 10 min data as is expected given the roughly lognormal distribution of 1 min rain rates.  The aver10 data has twice the sampling volume of the individual 10 min data sets. Since the DLR and UW disdrometers used different size categories to count drops, we could not average the raw counts per category and instead averaged the calculated R, W, and Z values for each time-synchronized sample to obtain the aver10 version of the data.

$ I never cared much about time matching, especially because the clock of our disdrometer PC is not very accurate. I know this makes part of the analysis questionable, but the time error should be less than a few minutes. Otehrwise the aver10 dataset will be not much usefull. Did you try a scatter plot or a time-series for the matched 10-(or 60-)minutes intervals?

All the numbers and plots I gave you before where not considering the UW computer failure , i.e. the DLR numbers data where with 3 half days more data. I now excluded those hours (before 1340 on 20Sep, after 0600 on 26Sep and from 0200 to 1000 on 21Oct) and redid HISTZ and HISTR, they change a little bit (see below). Also ZRSCAT is changed and the mean a for the DLR dataset reduces from 231 to 220. This again shows how sensitive the analysis is on  the dataset.

What exactly is combo10-60 ? Just taking all data and thus having about 26000 RDSD’s?

Did you use the extra DLR data for the incomplete days? $

# 4. Analysis

## a. Characteristics of samples from the two disdrometers

The calculated Z versus calculated R values for the accumulated 10 min samples from both disdrometers are shown in Figure ZRSCAT.  The points from both disdrometers are scattered relatively evenly throughout the plot indicating that the data from the two disdrometers likely represent two different samples from the same parent population. Overall there is a large scatter of up to 10 dBZ for a given rain rate. The frequency distributions of Z and R are very similar between the two instruments (Figure HISTZ and HISTR). There is a slight offset between the distributions of Z where the DLR disdrometer has higher frequencies for Z > 25 dBZ (Figure HISTZ). This offset is not apparent in the R distributions in Figure HISTR. Since the offset is larger in the D6 weighted Z than the ~D4 weighted R, we surmise it is due to a small residual error in the calibration of the size category boundaries for the larger drop sizes in one or both of the instruments.

$Are the Figures HISTZ and HISTR computed for the subset of times where the times exactly match (1243 10 min samples)? See above$

$ we need to justify that the data are close enough to each other that we can just combine the samples together into one big sample see above$

## b.  Calculation of Z-W and Z-R relations

The methods of calculating Z-R and Z-W relations from measured RDSD are almost as numerous as the number of papers that treat this subject. The resulting relationship can be very sensitive not only to the input data but also to the method by which it was calculated (Campos and Zawadzki 2000).

### i. Z-W

For the Z-W relations we use  a quadratic equation of the form W=qZ4/7  which simplifies into the linear equation:

log10(W)=log10(q)+(4/7)log10(Z)					[4]

The exponent 4/7 in the Z-W relation is obtained as follows. We approximate the RDSD as an exponential distribution, N(D)=Noe−ΛDdD for D from 0 to infinity where No is a constant, and integrate the definite integral forms of [1] and [2]. The general formula W=qZs is then translated into:

Noπ!Λ4=qNo6!Λ7s							[5]

We note that setting s=4/7 will cancel the  terms and remove the direct dependency of q on W.  Following Doelling et al.’s (1998) methodology, we determine a value of q for each sample of  the population using q=W/(Z4/7) .

The plot of log10(q) versus log10(W) (Figure ZMa) illustrates that log10(q) values are uncorrelated with W and vary between approximately 0.3 to 30 q units.  The sloping lower edge of the cloud of points is an artifact of the thresholding on 0.2 mm/hr rain rate. Lines of constant rain rate are roughly parallel to the lower right edge. The narrower distribution of q values for higher  rain rates is expected since the higher rain rate samples have a larger number of drops and less statistical sampling error than the lighter rain rate samples (see Section 3 and Table 2). The distribution of q is approximately lognormal (Figure ZMb) and the distribution of log10(q) for this data set is close to Gaussian (Fig. ZMc). We use the mean log10(q) value to obtain the best estimate and  1 standard deviation () of log10(q) as an assessment of the uncertainty (Table 3). The bottom half of Table 3 shows the equivalent values in q units. Since   1 standard deviation of log10(q) is not symmetric in q, we have indicated – as the 16th percentile and + as the 84th percentile.  Figure ZMd and the biases in Table 3 provide information on how well [4] estimates liquid water content from Z compared to liquid water content calculated from the RDSD in [2]. Cumulative bias is estimatedcalculated. Average bias is (estimatedcalculated)N.  While the spread of points around the 1:1 line  in Figure ZMd is wide, there is no bias to the cumulative estimate based on [4].  Individual estimates of W for dependent data will have an average positive bias of 15-18%. The statistics for the aver10 data set are nearly identical to the combo10 data set. The difference in the mean values between the combo10 and combo60 data is larger than the standard error of the mean (σN) but its physical significance is difficult to assess. The shift in the combo60 mean toward lower values is consistent with a reduction in the low bias of calculated Z relative to W associated with a smaller sampling error. In addition to smaller sampling error , combo60 data has smaller sample size $ ?larger? $ and larger errors associated with mixing rain patches compared to combo10.  Also short lasting convective ($?) rain events are not included into the dataset. A much larger data set than obtained during MAP would be needed to be able to quantify the relative contributions of these sources of uncertainty to the difference in mean q.  $ I’m not so fit in statistics, how is r² mathematical defined?

### ii. Z-R

Calculation of rain rate requires particle fall speed V(D,T,P) (see [3]). Even when T and P are assumed (here we use T = 20°C and P = 1013.25 hPa), empirical expressions for fall speed (Berry and Pranger 1974) do not have a simple functional form amenable to a definite integral solution for R. For the Z-R relation, we assume a quadratic equation of the form Z=aR1.5 which simplies to the linear equation:

log10(Z)=log10(a)+(1.5)log10(R)					[6]

The fixed exponent of 1.5 for the Z-R relation was originally proposed by Smith and Joss (1997) based on empirical studies and has been tested with multi-year samples of disdrometer data by Doelling et al. (1998) and Steiner and Smith (2000). As indicated in Figure ZRSCAT the Z-R relations are Z = 220R1.5 for the DLR data and Z = 201R1.5 for the UW data. When using a linear regression for the data the resulting Z-R relations are Z = 221R1.48 (DLR) and Z = 208R1.44 (UW). Therefore for the described dataset the assumption of the exponent to be 1.5 is justified.

The values of the coefficient a as a function of rain rate for each of the 10 min samples in combo10 are shown in Figure ZRa.  If there were distinct a values for lighter versus heavier precipitation, it would manifest in the scatter plot as discernably different populations of points as a function of R.  Instead, we have one widely scattered population of a values centered roughly between log(a) of 2 to 2.7.  As in Figure ZMa, there is a narrower distribution of a values for higher rain rates >5 mm/hr compared to <5 mm/hr since the higher rain rate samples have less statistical sampling error.

Similar to the characteristics of the distribution of q, the distributions of a is approximately lognormal (Figure ZRb) while log10(a) is roughly normal (Figure ZRc).

Similar to the procedure used to obtain the Z-W relation, we compute the mean value and standard deviation of log10(a) and their equivalent values in a  (Table 4). Again, the statistics for the combo60 data are shifted toward higher a values which is consistent with a reduction in the low bias of calculated Z relative to calculated R associated with a smaller sampling error. The fall velocity factor in R likely has a compensating effect for some types of errors as the  biases in Table 4 are slightly smaller than in Table 3 such that an individual estimate of R for dependent data will have an average positive bias of ~10%.

$ What is about the discussion of the „tourist“ and „hydrologist“ weighting of the histogram? Of course it makes the results more questionable, since we have one more point for complexity, but it also shows one problem more for defining a proper Z-R relation. $

### c. Uncertainties and their impact

### A recommendation to use a particular Z-W or Z-R relation is not truly complete without information on how well the suggested relations perform on independent data.  The nature of errors associated with these relations makes sample size particularly important and it is not uncommon for the entire available data set to be used to estimate the Z-W or Z-R relation even in multi-year data sets (e.g. Doelling et al. 1998, Steiner and Smith 2000).  Unfortunately, having used all the data to obtain our best estimate we have no independent data with which to test it.

The quality of the relation may be lowered if the sample size is reduced below some critical value.  If we split aver10 into two halves based on time, we have two independent data sets based on different sets of storms.  The mean coefficients vary slightly for the Z-R but not noticeably for the Z-W (right side of Tables 3 and 4) compared to the aver10 data set as a whole.  Application of the Z-R relation derived for one half of the data to the other half yields cumulative estimates equal to 105% of the RDSD calculated estimates.

By definition, 68.27% of the samples in the population fall within . The impact of applying the relations corresponding to the   q and a values are shown in Tables 5 and 6. For comparison,  the typical error in R associated with not correcting for the variation of the profile of reflectivity between the lowest radar measurement and the ground is 3 dB (factor of 2) in the Alps (Germann and Joss 2002).

# 5. Conclusions

$ say something about the Z-M relation I guess there are not many other papers on  Z-M relations, I have to check with my collegue, he was using one relation. Again we should point out the limitation on rain drops and not cloud drops$

Examination of RDSD data obtained from two Joss-Waldvogel disdrometers deployed at Locarno-Monti during MAP yields information on the relative magnitude of the RDSD variation component of error in mapping rainfall from observed reflectivity. Errors in 30 day rainfall accumulation due to RDSD variations in independent data are within 5% while errors in individual rain rates can be 64%-155% (Table 6). The relative sizes of these errors are small compared to several other known error sources in rainfall mapping from radar data and emphasize the importance of correcting overall biases with proper radar calibration and biases as a function of range using procedures to account for the variations in the vertical profile of precipitation from the radar measurement to the ground.

The combined 10 min accumulation (combo10) disdrometer data set mean relation of Z=216R1.5 is bracketed by a lower bound of Z=112R1.5 and an upper bound of Z=418R1.5.  These bounds encompass the 60 min accumulation (combo60) mean relationship and all the Z-R relations used by the national weather services within the MAP-SOP domain--Austria, France, and Italy Z=200R1.6 (Marshall and Palmer 1948) $ I know this value can not be found in M&P 48, but everyone cites it in this way. Klaus Beheng told me that Z=200R1.6 can be found in M&P 54 $,  Germany Z=256R1.42 (Aniol et al., 1980), and Switzerland Z=316R1.5 (Joss et al., 1998).   Since it is unlikely that variations in RDSD follow national boundaries, it would be useful to create a merged rainfall product based on quality controlled radar data for the MAP domain using a single Z-R relationship. From a qualitative standpoint, the exact relation used is not critical as a 5 dBZ difference will translate into a 105%, 125%, 115%, and 115% difference in R for the Marshall-Palmer, Anoil et al., Joss et al. and MAP Z-R relations respectively. As errors in rain rate at a particular point estimated from radar data are can be large (Figure ZRd, Table 6), comparisons with other data sets and numerical models are best done with areal averages or accumulations.

Acknowledgements

The authors gratefully acknowledge the collaboration of Jürg Joss and Urs Germann of the Swiss Meteorological Agency in the collection and analysis of the DLR and UW disdrometer data at Locarno-Monti. The second author also thanks MAP Principal Investigator Robert A. Houze, Jr. for his support and encouragement$ basic DLR funding, not necessary to be acknowledged $. The work of the second author was supported by U.S. NSF grant ATM-9817700 and NASA TRMM grant NAG5-9750.

# Appendix

The dead-time correction for the Joss-Waldvogel disdrometer accounts for the small drops of the RDSD which are not measured during the fraction of a second immediately after a large drop hits the instrument sensor (styrofoam cone).  A drop in the ith channel produces a dead time for all k channels where Dk < 0.85Di. The formula was developed by Joss and Waldvogel and is described in detail by Sauvageot and Lacaux (1995). It is included here to correct a typographical error in their paper. $in Sheppard & Joe 1994 is the correct one, so we should use this reference, and therefore could ommit the appendix)

Ni=Niexp0.035TDk=0.85DiD20Nk×logDk(0.85(Di−0.25))

where Ni is the number of drops in channel i without correction, Ni is the number of drops in channel i with correction, T is the sampling time in seconds, and log is the natural logarithm.

We applied the dead-time correction to the time accumulated RDSD rather than to each 1 min RDSD $ not in my program !!! $. The formula above does not correct the drop counts for size categories where no drops were measured (i.e. Ni=0). By accumulating the RDSD over time we increase the probability of detecting at least a few drops in each of the small size categories and can hence reduce this potential source of error. $ This is not so important for the Locarno measurements, I think they have considerable number of small drops in most of the RDSD $

# References

Aniol, R., J. Riedl, and M. Dieringer, 1980: Über kleinräumige und zeitliche Variationen der Niederschlagsintensität. Meteorol. Rdsch., 33, 50-56.

Austin, P. M., 1987: Relation between measured radar reflectivity and surface rainfall. Mon. Wea. Rev., 115, 1053-1070.

Bougeault, P., P. Binder, A. Buzzi, R. Dirks, J. Kuettner, R. Houze, R. B. Smith, R. Steinacker, and H. Volkert, 2001: The MAP Special Observing Period. Bull. Amer. Meteor. Soc., 82, 433-462.

Berry, E. X., and M. R. Pranger, 1974: Equations for calculating the terminal velocities of water drops. J. Appl. Meteor., 13, 108-113.

Bringi, V. N., and V. Chandrasekar, 2001: Polarimetric Doppler Weather Radar: Principles and Applications. Cambridge University Press, Cambridge, pp. 636.

Campos, E. and I. Zawadzki, 2000: Instrumental uncertainties in Z-R relations. J. Appl. Meteor., 39, 1088-1102.

Doelling, I. G., J. Joss, J. Riedl, 1998: Systematic variations of Z-R relationships from drop size distributions measured in northern Germany during seven years. Atmos. Res. 47-48, 635-649.

Doviak, R. J., and D. S. Zrnic, 1993: Doppler radar and weather observations, Academic Press, New York, 562 pages.

Frei, C. and C. Schär, 1998: A precipitation climatology of the Alps from high-resolution rain-gauge observations. Int. J. Climatol. 18, 873-900.

Germann, U.  and J. Joss, 2002: Meso-beta profiles to extrapolate radar precipitation measurements above the Alps to the ground. J. Appl. Meteor., in press.

Joss, J. and A. Waldvogel, 1967: Ein Spektrograph für Niederschlagstropfen mit automatischer Auswertung. Pure Appl. Geophys. 68, 240-246.

Joss, J., and E. G. Gori, 1976: The parameterization of raindrop size distributions. Riv. Ital. Geofis., 3, 275-283.

Joss, J., and E. G. Gori, 1978: Shapes of raindrop size distributions. J. Appl. Meteor., 7, 1054-1061.

Joss, J. and R. Lee, 1995: The application of radar-gauge comparisons to operational precipitation profile corrections. J. Appl. Meteor., 34, 2612-2630.

Joss, J., B. Schädler, G. Galli, R. Cavalli, M. Boscacci, E. Held, G. Della Bruna, G. Kappenberger, V. Nespor, and R. Spiess, 1998: Operational Use of Radar for Precipitation Measurements in Switzerland. vdf Hochschulverlag AG an der ETH Zürich, 108 pp.

Kostinski, A. B. and A. R. Jameson, 1997: Fluctuation properties of precipitation. Part I: On deviations of single-size drop counts from the Poisson distribution. J. Atmos. Sci., 54, 2174-2186.

Marshall, J. S., and W. McK. Palmer, 1948: The distribution of raindrops with size. J. Meteor., 5, 165-166.

Rotunno, R. and R. Ferretti, 2002: Comparative analysis of rainfall in MAP Cases IOP2B and IOP8.  QJRMS submitted.

Sauvageot, H. and J.-P. Lacaux, 1995: The shape of averaged drop size distributions. J. Atmos. Sci., 52, 1070-1083.

$is this correct? YES $Sheppard, B. E., 1990: Effect of irregularities in the diameter classification of raindrops by the Joss-Waldvogel disdrometer. J. Atmos. Ocean. Tech., 7, 180-183.

Sheppard, B. E., and P. Joe, 1994: Comparison of raindrop size distribution measurements by a Joss-Waldvogel disdrometer, a PMS 2DG spectrometer, and a POSS Doppler Radar. J. Atmos. Ocean. Technol., 11, 874-887.

Smith, P. L., Z. Liu, and J. Joss, 1993: A study of sampling variability effects in raindrop size observations. J. Appl. Meteor., 32, 1259-1269.

Smith, P. L., and J. Joss, 1997: Use of a fixed exponent in “adjustable” Z-R relationships. Preprints, 28th Conference on Radar Meteorology, Austin, TX, Amer. Meteor. Soc. 254-255.

Steiner, M., and J. A. Smith, 2000: Reflectivity, rain rate, and kinetic energy flux relationships based on raindrop spectra. J. Appl. Meteor., 39, 1923-1940.

Vignal, B., G. Galli, J. Joss, and U. Germann, 2000: Three methods to determine profiles of reflectivity from volumetric radar data to correct precipitation estimates. J. Appl. Meteor., 39, 1715-1726.

Waldvogel, A., 1974:  The No jump in raindrop spectra.  J. Atmos. Sci., 31, 1067-1078.

Table 1. Daily sums of precipitation measured with the rain gauge and the two disdrometers. A mark indicated days where the disdromter is within 10% of the daily rainfall measuerd with the gauge. The indicated total sum is only for the complete days.

| Date | Gauge (mm) | DLR (mm) | DLR within 10% | UW (mm) | UW within 10 % |
| --- | --- | --- | --- | --- | --- |
| 20 Sep 99 | 92.0 | 97.7 | Y | 43.8 | incomplete |
| 21 Sep 99 | 46.0 | 44.1 | Y | 42.8 | Y |
| 25 Sep 99 | 87.6 | 84.4 | Y | 88.4 | Y |
| 26 Sep 99 | 130.5 | 126.4 | Y | 73.2 | incomplete |
| 27 Sep 99 | 30.0 | 31.9 | Y | 30.9 | Y |
| 28 Sep 99 | 65.9 | 62.1 | Y | 62.0 | Y |
| 30 Sep 99 | 21.6 | 21.3 | Y | 21.7 | Y |
| 02 Oct 99 | 17.7 | 18.1 | Y | 19.0 | Y |
| 03 Oct 99 | 64.7 | 61.6 | Y | 61.7 | Y |
| 17 Oct 99 | 0.0 | 0.2 |  | 0.2 |  |
| 18 Oct 99 | 0.6 | 0.8 |  | 0.8 |  |
| 19 Oct 99 | 0.7 | 0.8 |  | 0.9 |  |
| 20 Oct 99 | 13.6 | 13.0 | Y | 13.7 | Y |
| 21 Oct 99 | 53.9 | 50.0 | Y | 36.5 | incomplete |
| 22 Oct 99 | 6.2 | 4.8 |  | 7.1 |  |
| 23 Oct 99 | 47.8 | 45.1 | Y | 46.7 | Y |
| 24 Oct 99 | 40.7 | 38.9 | Y | 42.5 | Y |
| 25 Oct 99 | 17.8 | 17.8 | Y | 18.6 | Y |
| 26 Oct 99 | 0.3 | 0.3 | Y | 0.3 | Y |
| 30 Oct 99 | 1.0 | 1.0 | Y | 1.1 | Y |
| 03 Nov 99 | 5.9 | 4.9 |  | 5.2 |  |
| 04 Nov 99 | 29.2 | 26.4 | Y | 27.2 | Y |
| 05 Nov 99 | 1.2 | 1.3 |  | 1.3 | Y |
| 06 Nov 99 | 50.5 | 47.5 | Y | 49.8 | Y |
| 10 Nov 99 | 0.0 | 0.1 |  | 0.1 |  |
| 11 Nov 99 | 18.1 | 17.9 | Y | 18.3 | Y |
| 14 Nov 99 | 6.8 | 6.5 | Y | 6.6 | Y |
| 15 Nov 99 | 2.9 | 2.9 | Y | 3.2 | Y |
| 17 Nov 99 | 8.0 | 7.9 | Y | 7.9 | Y |
| 18 Nov 99 | 0.7 | 0.0 |  | 0.0 |  |
| Sum | 585.5 | 561.5 |  | 577.8 |  |

Table 2. RDSD data sample statistics. The total accumulation and average rain rates are calculated after the dead-time correction is applied. The UW data had sporadic dropouts due to a computer problem so the time periods of the DLR and UW data do not match exactly.

| Description | Rain rate category | # Samples | Total Accum (mm) | Aver. Rainrate (mm/hr) | Drop Counts per Sample (as measured, no dead-time correction) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | min | mean | max |
| DLR 10 min | All | 1432 | 835 | 3.5 | 116 | 3445 | 12601 |
|  | R < 1 | 504 | 44 | 0.5 | 116 | 1735 | 6998 |
|  | 1  R < 5 | 674 | 265 | 2.4 | 255 | 3912 | 12601 |
|  | 5  R 10 | 138 | 157 | 6.8 | 1425 | 5147 | 10502 |
|  | 10  R < 50 | 112 | 326 | 17.5 | 1192 | 6088 | 9580 |
|  | R  50 | 4 | 43 | 64 | 7140 | 7658 | 8120 |
| UW 10 min | All | 1310 | 731 | 3.3 | 128 | 4075 | 17566 |
|  | R < 1 | 487 | 43 | 0.5 | 128 | 2238 | 9828 |
|  | 1  R < 5 | 607 | 244 | 2.4 | 315 | 4561 | 17555 |
|  | 5  R 10 | 113 | 131 | 6.9 | 1533 | 6245 | 12068 |
|  | 10  R < 50 | 100 | 286 | 17.1 | 2485 | 7431 | 11228 |
|  | R  50 | 3 | 28 | 56.1 | 9662 | 10158 | 10905 |
| Aver 10 min | All | 1243 | 717 | 3.5 | 370 | 7706 | 29942 |
|  | R < 1 | 415 | 38 | 0.6 | 370 | 3998 | 15959 |
|  | 1  R < 5 | 613 | 243 | 2.4 | 513 | 8556 | 29942 |
|  | 5  R< 10 | 120 | 138 | 6.9 | 2984 | 11279 | 20040 |
|  | 10  R < 50 | 92 | 270 | 17.6 | 4414 | 13779 | 20574 |
|  | R  50 | 3 | 28 | 55.5 | 17047 | 17673 | 18632 |
| DLR 60 min | All | 269 | 831 | 3.1 | 735 | 18306 | 61638 |
|  | R < 1 | 82 | 42 | 0.5 | 735 | 8818 | 29364 |
|  | 1  R < 5 | 144 | 334 | 2.3 | 2883 | 20674 | 61638 |
|  | 5  R< 10 | 25 | 174 | 7 | 6298 | 23445 | 46477 |
|  | 10  R < 50 | 18 | 281 | 15.6 | 17018 | 35447 | 49893 |
|  | R  50 | 0 | - | - | - | - | - |
| UW 60 min | All | 245 | 729 | 3.0 | 1930 | 21807 | 96483 |
|  | R < 1 | 80 | 40 | 0.5 | 1930 | 12053 | 44142 |
|  | 1  R < 5 | 127 | 297 | 2.3 | 6299 | 23641 | 96483 |
|  | 5  R< 10 | 22 | 145 | 6.6 | 11150 | 31235 | 55131 |
|  | 10  R < 50 | 16 | 247 | 15.5 | 23127 | 43048 | 62608 |
|  | R  50 | 0 | - | - | - | - | - |

Table 3. Estimates of coefficient q and its uncertainties and biases in log10(W)=log10(q)+(4/7)log10(Z) and W=qZ4/7.  is standard deviation, r2 is ratio of explained variation to total variation (coefficient of determination).

|  |  | combo10 | aver10 | combo60 | first half aver10 | second half aver10 |
| --- | --- | --- | --- | --- | --- | --- |
| log10(q) | mean | 0.529 | 0.531 | 0.460 | 0.533 | 0.530 |
|  |  | 0.25 | 0.23 | 0.23 | 0.22 | 0.25 |
|  | median | 0.527 | 0.532 | 0.458 | 0.543 | 0.507 |
|  | r2 | 1.05 | 1.02 | 1.09 | 1.1 | 0.93 |
|  | cumulative bias | 1.00 | 1.0 | 1.0 | 1.0 | 1.0 |
|  | average bias | 1.01 | 1.01 | 1.01 | 1 | 1.01 |
| q | mean | 3.4 | 3.4 | 2.9 | 3.4 | 3.4 |
|  | 16th percentile | 1.9 | 2 | 1.7 | 2.1 | 1.9 |
|  | 84th percentile | 6 | 5.8 | 4.9 | 5.6 | 6 |
|  | r2 | 1.05 | 1.02 | 1.09 | 1.1 | 0.93 |
|  | cumulative bias | 1.05 | 1.04 | 1.05 | 1.07 | 0.98 |
|  | average bias | 1.18 | 1.15 | 1.16 | 1.14 | 1.17 |

Table 4. Estimates of coefficient a and its uncertainties and biases in log10(Z)=log10(a)+(1.5)log10(R) and Z=aR1.5.

|  |  | combo10 | aver10 | combo60 | first half aver10 | second half aver10 |
| --- | --- | --- | --- | --- | --- | --- |
| log10(a) | mean | 2.335 | 2.334 | 2.428 | 2.324 | 2.344 |
|  |  | 0.29 | 0.27 | 0.27 | 0.25 | 0.28 |
|  | median | 2.332 | 2.331 | 2.427 | 2.297 | 2.364 |
|  | r2 | 1.09 | 1.07 | 1.11 | 1.09 | 1.06 |
|  | cumulative bias | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  | average bias | $ | $ | $ | $ | $ |
| a | mean | 216 | 216 | 268 | 211 | 221 |
|  | 16th percentile | 112 | 116 | 144 | 117 | 116 |
|  | 84th percentile | 418 | 400 | 499 | 379 | 422 |
|  | r2 | 1.09 | 1.07 | 1.11 | 1.09 | 1.06 |
|  | cumulative bias | 1.07 | 1.05 | 1.06 | 1.08 | 1.02 |
|  | average bias | 1.1 | 1.09 | 1.09 | 1.08 | 1.09 |

Table 5. Impact of  standard deviation in coefficient q in W=qR4/7 compared to combo10 mean value of 3.4.

| Coefficient a value | 1.9 | 3.4 | 6 |
| --- | --- | --- | --- |
| % difference in W estimated from Z | 56% | 100% | 176% |
| Difference in dBZ estimated from W | -4.4 | 0 | 4.3 |

Table 6. Impact of  standard deviation in coefficient a in Z=aR1.5 compared to combo10 mean value of 216.

| Coefficient a value | 112 | 216 | 418 |
| --- | --- | --- | --- |
| % difference in R estimated from Z | 155% | 100% | 64% |
| Difference in dBZ estimated from R | 2.3 | 0 | -2.4 |

Figure Captions

Figure ZRSCAT. Plot of calculated Z versus calculated R from 10 minute accumulated DLR and UW disdrometer RDSD samples. Solid lines indicate Z-R relation based on mean coefficient a values for each data set (DLR: Z=220R1.5 , UW: Z=201R1.5 ).  Dotted line indicates mean value for combo10 data set, Z=216R1.5.  $note dBZ should be on x axis to make more compatible with Doelling et al’s Figure 1. and replot lines to correspond to our final Z-R relations in Table A color figure is 500 pound, (~750 dollar ?)$

Figure HISTZ. Frequency distribution of calculated Z values for 10 minute accumulated DLR and UW disdrometer RDSD samples.

Figure HISTR. Frequency distribution of calculated R values for 10 minute accumulated DLR and UW disdrometer RDSD samples.

Figure ZM. a) Plot of RDSD calculated rain rate versus coefficient q in W=qZ(4/7). b) Frequency distribution of q, c) Frequency distribution of log10(q), d) Plot of RDSD calculated W versus estimated W using W=3.4Z(4/7) and calculated Z.

Figure ZR. a) Plot of RDSD calculated rain rate versus coefficient a in W=qZ(4/7). b) Frequency distribution of a, c) Frequency distribution of log10(a), d) Plot of RDSD calculated R versus estimated R using Z=216R1.5 and calculated Z.

? See Joss and Lee (1995), Joss et al. (1998), Vignal et al. (2000),and Germann and Joss (2001) for detailed discussion of these methods as they are applied by the MeteoSwiss to operational radar data.

The mean diameter of this 20th size category representing the largest drops over a particular size has the largest uncertainty compared to the other 19 size categories which have both minimum and maximum diameter limits.

The measurement accuracy of the MeteoSwiss rain gauge is $0.1 mm corresponding to the rainfall associated with a single tip of this tipping-bucket type gauge.

This processing method differs from other methods where consecutive rainy minutes are processed into 10 min accumulated samples.

These counts are raw drop counts (not dead-time corrected) as measured by the disdrometer rather than concentrations.

A Gaussian distribution of log10(q) is not generally true, especially for smaller sample sizes.

Doelling et al. (1998) used the median rather than the mean.

Although we are interested in obtaining a relation to transform observed Z into estimated R, we will follow the convention of describing this relation in terms of Z=aRb so our  results can be more readily compared to those reported by other investigators.