# Relation between radar reflectivity and rainfall rate during the MAP-SOP

Martin Hagen* and Sandra Yuter**

*  Deutsches Zentrum für Luft- und Raumfahrt (DLR), Institut für Physik der Atmosphäre, Weßling, Germany

** University of Washington, Department of Atmospheric Sciences, Seattle, USA

Abstract

Introduction

Precipitation is a dominant parameter in the world weather and climate. However, it is also one of the parameters which are not easily measured or simulated by numerical weather prediction models. Besides rain gauges weather radars are the most suitable tool to measure precipitation. Radars have the advantage of providing high spatial and temporal resolution ofer large areas. This is necessary for any meteorological or hydrological application.

In radar meteorology the relationship between radar reflectivity factor (Z) and rain rate (R) (known as the Z-R relation) is used to estimate rainfall rate at the earth surface. However, the parameters of the Z-R relations depend on the shape of the drop-size distribution. Battan (1973) gives 69 relations for varying rain types, geogrpahic locations and wave-length of the radar. The selection of the right Z-R relation is therefore essential for accurate estimations of the rainfall rate by radar.

During the Special Observation Periode (SOP) of the Mesoscale Alpine Programme (MAP) (Bougeault et al, 2001) rainfall measurement with operational weather radars where used for forecasting and estimation of diurnal rainfall. Several different radars operated by several national weather services were used. For historical reasons different Z-R relations are used by the national radar operators. In Austria, France and Italy the Marshall and Palmer (1948) relation Z=200R1.6 is used. In Germany Z=256R1.42 (Aniol et al., 1980) is used, wheras in Switzerland Z=316R1.5 (Joss et al., ????) is used. The purpose of this paper is therefore to use data measured during the MAP-SOP at Locarno (Switzerland) to show the high variance of the Z-R relations during several rain events and to provide a Z-R relation which could be applied for further studies in the Lago Maggiore area.

.....

Instruments

There are several possibilities to estimate Z-R relations. One could use radar measurements above a rain gauge to estimate directly the parameters of the relation. This method suffers under the effects like differences in sampling volume, advection or evaporation of rain during the fall from the radar measuring volume down to the rain gauge. Radar measurements can be affected by attenuation or ground clutter. On the other hand this is closer to the anticipated later use of the Z-R relation.

Here the more common approach is selected. Measurements of drop-size distributions are used to estimate the rainfall rate and reflectivity factor. To measure the drop-size distribution we use two Joss-Waldvogel disdrometer (Joss and Waldvogel, 1969). This is the classical instrument to record raindrop size distributions. Even though it was designed some 35 years ago its simple electro-mechanical concept is still appropriate. The momentum of a raindrop falling at its terminal velocity on a styrofoam cone (area 50 cm²) is converted to an electrical impulse. This the height of this impulse is proportional to the diameter of the raindrop. For further analysis, the impulses are then grouped into 20 classes representing drop size classes.

The diameter range which can be identified by the disdrometer is in the order of 0.3 to 5 mm. Smaller drops do not produce a signal sufficient above the noise flour of the analysis unit. Larger particles are all grouped into the last of the 20 classes. Therefore the representativiness of the disdrometer for small drops (say below 0.6 mm) and for large drops (> 5 mm) is limited. Also at larger rain rates the detection effectivity of small drops is reduced due to the generation of enviornmental noise. In order to account for the dead-time of the sensor after a drop hit the cone a correction proposed by Waldvogel (described in Joe and Shepard, 199.) is applied to the drop counts.

Two disdrometers were used, one operated by the University of Washington (UW) and the other operated by the DLR Institut für Physik der Atmosphäre. Both were located close together at the site of the Osservatorio Ticinese of MeteoSwiss in Locarno-Monti.

The calibration of the disdrometer and its properties can be found in several recent publications like the ones by Kinnel (197.) or Joe and Shepard (199.).

The DLR disdrometer was only calibrated by adjusting the boundaries beween the drop size classes by a procedure similar to the one described by Shepard (). An further adjustment was performed in comparing the total rainfall sum to the one observed by a close rain gauge of MeteoSwiss. Joss (personal communication) found that the boundaries between the classes had to be reduced by 16% to match both instruments. However, this can not consider effects if the width of the individual classes are not exactly determined.

Recalibration of DLR disdrometer ????

The UW disdrometer ...

## Data evaluation

For the time periode 20 Sept. 1999 until 19 Nov. 1999 both instruments were operated simultaneous.  A total of about 13000 1-minute drop-size distributions were recorded by each of the instruments. First of all all spectra containing less than 20 drops were excluded. In a next step all spectra resulting in less than 0.2 mm/h rain rate were omited. Both filters were applied to exclude artifacts or spectra at the very begin of a rainfall periode.

A drop-size distribution is defined by the number of raindrops of a certain diameter (D in mm) class per volume (m3) as N(D) in mm-1m-3. The following integral parameters of the size distribution are used through the remainder of this text.

The rainfall rate is defined as the vertical flux of water through surface per time unit

R =

where R is the rainfall rate in mm/h, D the drop diameter (in mm), v(D) the terminal fall velocity (in m/s), and N(D) the number density of raindrops per size interval (in mm-1 m-3). The terminal fall velocity is parametrized by

v(D) =

(Atlas et al.,1973).

The 3rd moment of the size distribution is the liquid water content (in g m-3)

W =

The 6th moment of  the size distribution is the equivalent radar refelctivity factor in mm-6 m-3

Z =

assuming Rayleigh scattering, i.e. particles are much smaller than the wave length of the radar.

For the time interval 20 Sept. 1999 until 25 Oct. 1999 the total rainfall observed by the rain gauge was 514 mm, for the DLR disdrometer 451 mm and for the UW disdrometer 509 mm. Figure 1 shows daily precipitation sums for the disdrometers versus the raingauge. The two ouliers due to temporal failure of the UW disdrometer were not considered for the sums mentioned above. It shows that the DLR disdrometer in general underestimates the rainfall by a few percent. However, this is not a linear relationship and therefore requires more attention.

A further comparison was made considering the average drop-size distribution observed during this periode. Two further instruments were included, both a PARSIVEL optical disdrometer (Löffler-Mang and Joss, 2000). Figure 2 shows the comparison of the four instruments. In general an exponential drop-size distribution is expected for this long-time average. In the range of about 1 to 3 mm drop diameter the instruments agree fairly well. At smaller drops the instruments have less sensing efficiency, the J&W disdrometer due to environmental noise, the PARSIVEL due to optical problems. The ribles at the small end of the spectra of the DLR disdrometer are due to not exactly determined boundaries of the drop-size classes. The differences at the large drop end can explained by different calibration methods, but also the representativity of the large drops is very poor, only few of them are observed in each of the individual 1-minute drop-size distribution. Also shown is the corresponding drop-size distribution as it would follow from the parametrization according to Marshall and Palmer (1948) using N0 = 8000 mm-1 m-3 and lambda=0.41 R-0.21. The observations at Locarno show a much higher contribution of larger drops and a lower number of smaller drops compared to the exponential distribution according to Marshall and Palmer.

In order to derive Z-R relations reflectivity factor (Z) and rain rate (R) of drop-size distributions are computed and then through a linear regression the coefficients a and b of the following relation are estimated

Z = a Rb

Due to the nature of linear regressions the coefficients of a R = c Zd relation would not correspond mathematical directly to a and b. Even though for practical applications a R-Z relation would be more appropriate (rain rate is estimated from radar measurements) we stay with the classical definition of the Z-R relation.

Z and R are both moments of the drop-size distribution. Z is related to the 6th moment and R to the 3.7th moment, 3rd moment due to the volume and 0.7th moment due to a approximation of the terminal fall velocity by  Atlas and Ulbrich (1977). This results in a different behaviour of Z and R on the drop-size distribution. R is more sensitive to concentration changes at the small sizes whereas Z is more sensitive to changes at larger drops. This is why calibration plays an important role for the derivation of Z-R relations. Also the shape of the drop-size distribution is of relevance. Observations show that for long term measurements the drop-size distribution is close to an exponential one, like the ones shown in Figure 2. This is especially true for stratiform precipitation. Short term observations like the 1-minute disdrometer measurements can deviate considerable from an exponential one. Especially because of the poor sampling representativity and the stochastic nature of rain there is a wide scatter from spectrum to spectrum (e.g. Joss and Waldvogel, 1969). Figure 3 shows an example during 1 hour of stratiform precipitation. There is a large scatter around the averaged 1-hour distribution.

In this context the representativity of the drop size distributions is of great importance. At a rain rate of 1 mm/h the disdrometer observes about 320 rain drops. This corresponds to about 500 raindrops per mm³. According to a Marshall and Palmer distribution the concentration for drops larger than about 2 mm is less than 1. This indicates that they are only observed randomly. On the other side the radar measurement volume (1.3 e8 m³ at 60 km range, 1° beam width, 1µs pulse duration) contains about 6.5 e10 raindrops at the same rain rate which are sampled in less than 0.1 s. This corresponds to about 250 years of disdrometer rain measurements.

## Results

For DLR and UW disdro:

Z-R for 1 minute

Z-R for 1 hour

Fixed slope b=1.5

Cumulative frequency of a for b=1.5

## Conclusions

Acknowledgement: Ullrich Blahak for PARSIVEL Data, Jürg Joss for hospitality and discussions, Urs Germann for technical help.

Figure 1

Figure 2

Figure 3